Quick Reminder: Automatic Current Limit (aka ZEUS Circuit)

- Each output channel has built in over current protection.

- As the current increases the output voltage is reduced to prevent damage to the LST tube

- The limit circuit kicks in when the current increases beyond 
                       IKnee = (HVBrick – HVSet) / 500M

- Current configuration: HVBrick – HVSet = 1000 V -> IKnee = 2000 nA

- Possible adjustments

  - max output Voltage (Ultravolt module) 		10 KV

  - max internal voltage (resistor divider)		7.5 KV

  - max HVBrick  (low bits of DAC  set to 0)		7.3 KV

  - max IKnee with HVSet = 5.5 KV                           3600 nA

- Proposal

  - Set HVBrick to HVSet + 1500 V

  - IKnee (at 5.5 KV) = 3000 nA

  - Passed 100 hrs test at OSU