| Versatile Link Technical Specification, part 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System |  |  |  |  |  |
| Abstract This document describes the system level power, jitter and environmental specifications of the Versatile Link common project for optical transmissions in SLHC detectors. |  |  |  |  |  |
| Prepared by : A. Xiang SMU Physics 3215 Daniel Ave Dallas, TX 75275 [cxiang@smu.edu] |  |  | Checked by : T. Huffman A. Prosser J. Troska F. Vasey T. Weidberg J. Ye |  | Approved by: |
| History of Changes |  |  |  |  |  |
| Rev. No. | Date | Pages |  | Description of Changes |  |
| 0.1 0.2 0.3 0.4 0.5 0.6 0.7 0.8 0.9 1.0 1.1 1.2 1.3 1.4 1.5 1.6 1.7 1.8 | 19 Sep 2009 28 Oct 2009 10 Nov 2009 16 Dec 2009 15 Feb 2010 26 Apr 2010 30 Jun 2010 09 Nov 2011 07 Dec 2011 17 May 2011 12 Jun 2011 10 Aug 2011 31 Aug 2011 23 Sep 2011 08 Feb 2012 08 May 2012 24 May 2012 13 Jun 2012 |  |  | First draft version First draft per discussion Specify up and downlink Propose power budget table Distinguish VTRx and TRx Power budget table update Revision for discussion Power budget table update Add nomenclature Add jitter budget Change to specification format Modify text on temperature Format change Group review Add notes to tables Complete eye mask table Extend MM RMS spectral width Modified VTRx temperature range Modified VTRx temperature range |  |

Specification Tree

The hierarchy of the Versatile Link system specification is shown below.  The position of the present specification document is highlighted in bold.  Line items in italic will not result in specification documents but are shown to ease understanding of the structure.

EDMS Document Number

Part 1	System								1140664

Part 2	Components

Part 2.1	Front-end Transceiver					1140665

Part 2.1.1	Transmitter Optical Sub-Assembly (TOSA)		1141155

Part 2.1.2	Laser Driver						1141163

Part 2.1.3	Receiver Optical Sub-Assembly (ROSA)		1141157

Part 2.1.4	Trans-impedance Amplifier				1141160

Part 2.2	Back-end Components

Part 2.2.1	SFP+ Transceiver					1146246

Part 2.2.2	Parallel Optics						1146248

Part 2.3	Passive Optical Components

Part 2.3.1	Optical Fibres and Connectors			1146253

Table of Contents (not compulsory, can be removed)

1.

Introduction	5

2.	Nomenclature	6

3.	general specification	7

4.	environmental Specifications	8

4.1	radiation field	8

4.2	temperature range	8

4.3	magnetic field	8

4.4	EMI	8

5.	power budget	9

5.1	link penalties	10

5.2	radiation degradation	10

5.3	Transceiver customization	11

5.4	attenuation and insertion loss	11

6.	jitter budget	12

6.1	test points	12

6.2	jitter outputs and jitter contributions	12

7.	electrical and optical eye masks	14

8.	reliability Guideline	16

9.	glossary	18

10.	references	19

11.	Appedix	20

11.1	power penalty model	20

11.2	Components Radiation degradation	21

# Introduction

The Versatile Link project [1] aims to provide a multi-gigabit per second optical physical data transmission layer for the readout and control of High Luminosity LHC (HL-LHC) experiments. A point-to-point bidirectional system architecture is proposed for which components are currently being assessed and developed, as shown in Figure 1.

Figure 1: A point-to-point radiation hard optical link for HL-LHC

The link components consist of electro-optical transceivers, fiber cables and other passive components such as connectors and splitters. The versatile transceiver (VTRx) is placed on detector, at the front-end. The standard transceiver (TRx) is placed off detector, at the back-end. The nominal form factor of the transceivers is SFP+ (enhanced small form pluggable module) [2].

The VTRx modules contain the following custom developed or qualified parts: a laser diode inside an optical sub-assembly and a radiation tolerant laser driver (i.e., GBLD developed by the CERN GBT project) on the transmit side; a PIN diode with a radiation tolerant trans-impedance amplifier and limiting amplifier (i.e., GBTIA also supplied by GBT project) inside an optical sub-assembly on the receive side.

The TRx modules are commercial devices compliant to 10Gbps standards. When used in the Versatile Link, both VTRx and TRx are operated at the 4.8Gbps nominal data rate.

The link supports single-mode (SM) operation with a center-wavelength of 1310nm as well as multi-mode (MM) operation with a center-wavelength of 850nm. There are two variants of the VTRx and TRx modules corresponding to the SM and MM operations. The fiber cables are commercial SM and laser optimized MM fibers (OM3/OM4 fibers) correspondingly.

The link is naturally bi-directional. But matching the number of front-end to back-end (uplink) and back-end to front-end (downlink) transmissions is not a requirement. More data will likely transit from front-end to back-end. It is therefore possible to have multi-channel transmitters or receivers in the system.

# nomenclaure

The Versatile Link system can be decomposed into four general link variants as shown in Figure 2.1.  Transmission characteristics and device families of each are listed in Table 2.1.

Table 2.1 Versatile Link characteristics and device families

|  | Link Variants | Transmission | Note | VTRx | TRx1 |
| --- | --- | --- | --- | --- | --- |
| 2.1.1 | MM_VTx_Rx | MM uplink | 850nm, MMF OM3/OM4 | GBLD+850VCSEL | 10GBASE-SR 40GBASE-SR4 100GBASE-SR10 Enhanced SNAP12 Opto Engine |
| 2.1.2 | MM_Tx_VRx | MM downlink |  | GBTIA+GaAs PIN GBTIA+InGaAs PIN |  |
| 2.1.3 | SM_VTx_Rx | SM uplink | 1310nm, SMF | GBLD+1310FP GBLD+1310VCSEL | 10GBASE-LR 40GBASE-LR4 Enhanced SNAP12 Opto Engine |
| 2.1.4 | SM_Tx_VRx | SM downlink |  | GBTIA+InGaAs PIN |  |

Note 1: The SM array TRx and embedded TRx such as 40GBASE-LR4 and opto engines will be evaluted as they become commercially available (within 6 -12 months).

MM_VTx_Rx

SM_VTx_Rx

Front-End

MM_Tx_VRx

Post- Amp

Laser

Driver

GBLD

Laser

PIN/GBTIA

Laser

Post- Amp

Laser

Driver

Laser

PIN/GBTIA

Laser

SM_Tx_VRx

GBLD

Back-End

2.1.1

2.1.2

2.1.3

2.1.4

PIN/TIA

PIN/TIA

Figure 2.1 The four variants of Versatile Link with regard to system specifications

# general Specifications

Table 3.1 Versatile Link general specification

|  | Parameter | Symbol | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- | --- |
| 3.1.1 | Bit Rate | BR |  | 4.8 |  | Gbps |
| 3.1.2 | Bit Error Ratio1 | BER |  |  | 10-12 |  |
| 3.1.3 | Max. link length | LMAX | 150 |  |  | meter |

Note 1: Test pattern for the BER measurement should be reasonably stressful. Thus PRBS23-1 pattern or longer record length are preferred.

# Environmental Specifications

## Radiation Field

For each variant of the Versatile Link there are multiple technology choices, all of which may not be able to meet the strictest environmental requirements of the SLHC applications.   To balance cost, reliability and performance, the Versatile Link is developed in two radiation-tolerance classes. Table 4.1.1 shows the corresponding requirements. Radiation environment predictions at SLHC and irradiation guidelines are given in reference [3].

Table 4.1.1 Versatile link environmental requirements

|  | Tolerance level | Dose and fluence1(1Mev neutron equivalent) |
| --- | --- | --- |
| 4.1.1.1 | Calorimeter | 10 kGy 5 x 1014 n/cm2 |
| 4.1.1.2 | Tracker | 500 kGy 2 x 1015 n/cm2 1 x 1015 h/cm2 |

Note 1: The fluence level requirement depends on the particle type and energy. A scaling factor should be established if using different particles and energies for testing. For example, the tracker grade fluence test level would be  6 x 1015 n/cm2 for 20MeV neutron.

## Temperature Range

Front-end components, i.e., VTRx and the first meters of the fiber cable plant should meet the extended limit to as low as -30°C. Back-end component only need to comply with commercial standard for indoor operation. No special storage temperature and relative humidity specifications are foreseen and are therefore set to the lower limits of the constituting components. The link power and jitter margins shall be maintained throughout the temperature range.

Table 4.2.1 Component operation temperature range

|  | component | Min | Typ | Max | units |
| --- | --- | --- | --- | --- | --- |
| 4.2.1.1 | VTRx | -30 |  | 60 | °C |
| 4.2.1.2 | Passives | -30 |  | 70 | °C |
| 4.2.1.3 | backend | 0 |  | 70 | °C |

## Magnetic Field

4.3.1 - Magnetic field at front-end: 4T

Use of non-magnetic materials is mandatory for VTRx and front-end passive components.

## EMI

4.4.1 - EMI requirement for VTRx: TBD.

4.4.2 - EMI requirement for back end: FCC class 2.

# Power budget

Power budgets for tracker-grade and calorimeter-grade Versatile Links are shown in table 5.1 and 5.2.  All power levels are specified to meet target performance of bit error ratio better than 10-12.

Table 5.1 Tracker-grade Versatile Link power budget

|  | MM_VTx_Rx | MM_Tx_VRx | SM_VTx_Rx | SM_Tx_VRx |
| --- | --- | --- | --- | --- |
| Min. Tx OMA | -5.2 dBm | -1.6 dBm | -5.2 dBm | -3.6 dBm |
| Max. Rx sensitivity | -11.1 dBm | -13.1 dBm | -12.6 dBm | -15.4 dBm |
| Power budget | 5.9 dB | 11.5 dB | 7.4 dB | 11.8 dB |
| Fiber attenuation | 0.6 dB | 0.6 dB | 0.1 dB | 0.1 dB |
| Insertion loss | 1.5 dB | 1.5 dB | 2.0 dB | 2.0 dB |
| Link penalties | 1.0 dB | 1.0 dB | 1.5 dB | 1.5 dB |
| Tx radiation penalty | 0 dB | - | 0 dB | - |
| Rx radiation penalty | - | 5.4 dB | - | 5.4 dB |
| Fiber radiation penalty | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
| Margin1 | 1.8 dB | 2.0 dB | 2.8 dB | 1.8 dB |

Note 1: Error coding scheme, for example, the GBT coding can result in in an additional 2-3 dB gain in margin.

Table 5.2 Calorimeter-grade Versatile Link power budget

|  | MM_VTx_Rx | MM_Tx_VRx | SM_VTx_Rx | SM_Tx_VRx |
| --- | --- | --- | --- | --- |
| Min. Tx OMA | -5.2 dBm | -3.2 dBm | -5.2 dBm | -5.2 dBm |
| Max. Rx sensitivity | -11.1 dBm | -13.1 dBm | -12.6 dBm | -15.4 dBm |
| Power budget | 5.9 dB | 9.9 dB | 7.4 dB | 10.2 dB |
| Fiber attenuation | 0.6 dB | 0.6 dB | 0.1 dB | 0.1 dB |
| Insertion loss | 1.5 dB | 1.5 dB | 2.0 dB | 2.0 dB |
| Link penalties | 1.0 dB | 1.0 dB | 1.5 dB | 1.5 dB |
| Tx radiation penalty | 0 dB | - | 0 dB | - |
| Rx radiation penalty | - | 2.5 dB | - | 2.5 dB |
| Fiber radiation penalty | 0.1 dB | 0.1 dB | 0 dB | 0 dB |
| Margin1 | 2.7 dB | 4.2 dB | 3.8 dB | 4.1 dB |

Note 1: Error coding scheme, for example, the GBT coding can result in in an additional 2-3 dB gain in margin.

Link penalties, radiation degradation, fibre attenuation and insertion loss are detailed in the following sections.

## Link penalties

The 10GbE link model [5], a physical layer power calculation model developed by the IEEE 802 standard working group, is adopted to specify the Versatile Link penalties [6]. Appendix 11.1 illustrates the worksheets with input and output of MM and SM link penalty simulations. Principal link parameters are summarized in table 5.1.1.  The maximum link penalties are 1.0 dB for MM operation and 1.5 dB for SM operation.

Table 5.1.1 Key characteristics in support of Versatile Link model simulation

|  | Parameter | MM_VL | SM_VL |
| --- | --- | --- | --- |
|  | Data rate (Gbps) | 5 | 5 |
|  | Link length (m) | 150 | 150 |
|  | Center wavelength (nm) | 850 | 1310 |
| 5.1.1.1 | RMS spectral width (nm) | 0.65 | 4.0 |
| 5.1.1.2 | Optical rise/fall time (ps) | 751 | 751 |
|  | Relative intensity noise (dB/Hz) | -130 | -130 |
|  | Mode partition noise coefficient | 0.3 | 0 |
|  | Reflection coefficient | 0 | 0.62 |
| 5.1.1.3 | Link penalty (dB) | 1.0 | 1.5 |

Note 1. In both MM and SM links, power penalties are sensitive to the optical rise/fall time. The value of 75 ps used in simulation is larger than component specification to ensure a conservative estimation.

Note 2. The same reflection coefficient is set for DFB based TRx and FP based VTRx.

## RadiatioN degradation

While operating in the radiation environment described in section 4, link components situated at the front-end (i.e., VTRx and fiber) shall maintain the target bit error rate performance through the designed life cycle of the detectors.  Power penalties allocated to the cumulative degradations are listed in table 5.2.1. Appendix 11.2 describes the derivations of the maximum values induced by radiation.

Table 5.2.1. Power penalties allocated to cumulative radiation degradation

|  | Component | Radiation degradation in Tracker-grade environment (dB) | Radiation degradation in Calorimeter-grade environment (dB) |
| --- | --- | --- | --- |
| 5.2.1.1 | MM_VTx1 | 0 | 0 |
| 5.2.1.2 | SM_VTx1 | 0 | 0 |
| 5.2.1.3 | MM/SM_VRx2 | 5.4 | 2.5 |
| 5.2.1.4 | MMF3 | 1.0 | 0.1 |
| 5.2.1.5 | SMF | 1.0 | 04 |

Note 1. Transmitter output power shall not decrease with radiation. Adequate drive current shall be provided to compensate radiation degradations that include raised threshold and reduced slope efficiency of laser.

Note 2. Receiver penalty includes the effect of reduced responsivity and increased dark current of the PIN diode.

Note 3. Radiation induced attenuation in multi-mode fibers shall represent the maximum allowed value for highest dose rate and lowest temperature.

Note 4. Radiation induced attenuation in single-mode fibers used in calorimeter applications is negligible (<0.05dB) compared to other degradations and losses.

## Transceiver customization

VTRx transceiver power levels shall comply with table 5.3.1. Power requirements of TRx transceiver for calorimeter applications are compatible with commercial standards, while power levels of  tracker TRx transmitters are higher than that of the commercial standards. Vendors who provide these customized parts shall comply with table 5.3.1.

Table 5.3.1 Transceiver power levels

|  | Component | Tx_OMA_min (dBm) | Rx_sensitivity_max (dBm) |
| --- | --- | --- | --- |
| 5.3.1.1 | Tracker_MM_VTRx | -5.2 | -13.1 |
| 5.3.1.2 | Tracker_SM_VTRx | -5.2 | -15.4 |
| 5.3.1.3 | Calorimeter_MM_VTRx | -5.2 | -13.1 |
| 5.3.1.4 | Calorimeter_SM_VTRx | -5.2 | -15.4 |
| 5.3.1.5 | Tracker_MM_TRx | -1.6 | -11.1 |
| 5.3.1.6 | Tracker_SM_TRx | -3.6 | -12.6 |
| 5.3.1.7 | Calorimeter_MM_TRx | -3.2 | -11.1 |
| 5.3.1.8 | Calorimeter_SM_TRx | -5.2 | -12.6 |

## Attenuation and insertion loss

5.4.1 - The fibre attenuation is based on the maximum link distance of 150 meters. 0.6 dB is allocated to MMF with 3.5 dB/km of attenuation, and 0.1 dB is allocated to SMF with 0.4 dB/km of attenuation.

5.4.2 - The insertion loss is specified for a number of connections and splices. For example, 1.5dB is allocated to MM link in support of three connector pairs or splices, with an insertion loss of 0.5 dB each. 2.0 dB is allocated to SM link in support of three connections of insertion loss of 0.65 dB each.

# jitter budget

## Test points

Jitter allocation is specified at test points (interoperability points) along a complete data link. There are four test points defined in the Versatile Link, as illustrated in figure 6.1.1. The up- and down-stream serializing and deserializing ICs interfacing with the Versatile Link are likely implemented in FPGAs and ASICs such as GBTX. The rest of this specification assumes that the output requirement of up-stream IC and the input requirement of down-stream IC comply with industrial standards [8][9]. The following specification also assumes that the measurement of the jitter content at all test points complies with stressed signal conditioning and reference receiver requirements as stated in reference [9]. Only jitter output and link component contributions are specified in the following. Jitter tolerance requirements will be addressed in future additions.

Figure 6.1.1 Test points in Versatile Link for jitter specification

## jitter outputs and jitter contributions

The output jitter specification along the link test points is listed in table 6.2.1, where total jitter (TJ) is broken down into deterministic jitter (DJ) and random jitter (RJ).

DJ is measured as peak to peak value and RJ is measured as RMS value (one σ of a Gaussian distribution. At target BER of 10-12, the confidence interval required is 14.1 σ).

TJ = DJ + RJ = DJ + 14.1* RJrms

In addition, we derive the Versatile Link components jitter contributions in the following equations.

DJoutput = DJinput + DJcontribution

RJoutput = SQRT ( RJinput ^2 + RJcontribution ^2)

Table 6.2.1. Jitter budget and component contribution

|  |  | TJ (UI) | DJ (UI) Peak-peak | RJ (UI) 14.1σ | Comment |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1 | TP1 | 0.26 | 0.14 | 0.12 | PLL, serdes |
| 6.2.1.2 |  |  | 0.12 | 0.13 | OE transmitter contribution At 4.8 Gbps, the OE transmitter contributed jitter is under 25 ps DJp-p, 1.9 ps RJrms |
| 6.2.1.3 | TP2 | 0.44 | 0.26 | 0.18 |  |
|  |  |  | 0.02 | 0.09 | Fiber contribution |
| 6.2.1.4 | TP3 | 0.48 | 0.28 | 0.20 |  |
| 6.2.1.5 |  |  | 0.14 | 0.20 | EO receiver contribution At 4.8 Gbps, the EO transmitter contributed jitter is under 29 ps DJp-p, 2.9 ps RJrms |
| 6.2.1.6 | TP4 | 0.70 | 0.42 | 0.28 |  |
|  |  |  | 0.13 | 0.12 | Serdes, CDR |
|  |  |  | 0.15 |  | tolerance margin |
|  | Total | 1.00 | 0.70 | 0.30 |  |

# electrical and optical eye masks

In addition to the amplitude, jitter and sensitive parameter requirements listed in section 5 and 6, the input and output electrical/optical waveforms at the link test points shall also comply with the eye mask specifications to assure target BER performance.

The following shows the eye mask specifications for VTRx and TRx modules. Testing conditions are similar to those stated in reference [8].

Figure 7.1 Electrical input mask

Figure 7.2 Optical output mask

Figure 7.3 Electrical output mask

Table 7.1. Electrical and optical eye mask for VTRx and TRx

| 7.1.1 | VTRx Elec. Input Mask | X1 | X2 | X3 | Y1 | Y2 | Y3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Value | 0.13 | 0.4 |  | 50 | 600 |  |
|  | Unit | UI | UI |  | mV | mV |  |
| 7.1.2 | VTRx Opto. Output Mask | X1 | X2 | X3 | Y1 | Y2 | Y3 |
|  | Value | 0.25 | 0.4 | 0.45 | 0.25 | 0.28 | 0.4 |
|  | Unit | UI | UI | UI |  |  | Y1 |
| 7.1.3 | VTRx Elec. Output Mask | X1 | X2 | X3 | Y1 | Y2 | Y3 |
|  | Value | 0.35 | 0.5 |  | 125.5 | 250 |  |
|  | Unit | UI | UI |  | mV | mV |  |
| 7.1.4 | TRx Elec. Input Mask | X1 | X2 | X3 | Y1 | Y2 | Y3 |
|  | Value | 0.13 | 0.4 |  | 325 | 800 |  |
|  | Unit | UI | UI |  | mV | mV |  |
| 7.1.5 | TRx Opto. Output Mask | X1 | X2 | X3 | Y1 | Y2 | Y3 |
|  | Value | 0.25 | 0.4 | 0.45 | 0.25 | 0.28 | 0.4 |
|  | Unit | UI | UI | UI |  |  | Y1 |
| 7.1.6 | TRx Elec. Output Mask | X1 | X2 | X3 | Y1 | Y2 | Y3 |
|  | Value | 0.35 | 0.5 |  | 150 | 425 |  |
|  | Unit | UI | UI |  | mV | mV |  |

# Reliability Guideline

Reliability must be demonstrated against targeted environmental stresses. These include internally and externally induced effects on optical coupling, thermal path and mechanical strain. An example system failure rate budget is assigned as follows:

Table 8.1. Example system failure rate allocation.

|  | Component | Type | FIT1 |
| --- | --- | --- | --- |
| 8.1.1 | VTRx | On detector transceiver laser with driver/PIN with amplifier both 850nm and 1310nm | 2503 |
| 8.1.2 | TRx | Off detector transceiver module Both 850nm and 1310nm | 2002 |
| 8.1.3 | Other components | Connector Fiber cable | 25 25 |
| 8.1.4 | Link | duplex | 5004 |

Note 1: Failure is defined as link BER exceeding 10-12. FIT is one failure in billion (109) device-hours.

Note 2: TRx FIT of 200 is allocated after Telcordia (SR-332, issue 2, 2006) .

Note 3: VTRx FIT is allocated assuming similar sub-device reliability, reduced part count, and extra margin for customized assembly.

Note 4: For 10 years of detector operation, 500 FIT is equivalent to 5% channel failure. Some of which (the front-end portion) are not accessible.

# Glossary

VTRx – Front-end optical transceiver module, installed on detector

TRx – Back-end optical transceiver module, installed in counting room

MM – multi-mode operation

MMF – multi-mode fibre

SM – single-mode operation

SMF – single-mode fibre

# References

[1]	 “The Versatile Link, A Common Project for Super-LHC”, L. Amaral et al., JINST 4 P12003, 2009

[2]	“SFF-8431 Specifications for Enhanced Small Form Factor Pluggable Module SFP+ Revision 4.1”,

ftp://ftp.seagate.com/sff/SFF-8431.PDF

[3]  Joint ATLAS-CMS working group on optoelectronics for SLHC, report from sub-group B, Optical system irradiation guideline, https://edms.cern.ch/document/882783/2.6

[4]  “Versatile Transceiver Developments”, J. Troska et al, JINST 6 C01089, 2011

[5]  The 10G Ethernet Link Model

http://ieee802.org/3/ae/public/adhoc/serial_pmd/documents/10GEPBud3_1_16a.xls

[6]  “Link model simulation and power penalty specification of the versatile link system”, A. Xiang et al., JINST 6 C01088, 2011

[7]  “The radiation hardness of specific MM and SM optical fibers at -25°C beyond a full SLHC dose to a dose of 500 kGy(Si)”, T. Huffman et al., JINST 5 C11023, 2010

[8]  802.3ae-2008, clause 52,

http://standards.ieee.org/getieee802/download/802.3-2008_section4.pdf

[9]  FC-PI-5, revision 0.01, section 6,

ftp://ftp.t11.org/t11/pub/fc/pi-5/09-392v0.pdf

# Appendix

## power penalty model

Link model spread sheet download link in reference [5]

11.1.1. Link penalty calculation of MM_VL

11.1.2. Link penalty calculation of SM_VL

## components radation degradation

11.2.1. PIN degradation with cumulative radation at a glance

Detail results are discussed in reference [4]

|  | Worst case is for 6×1015 cm-2 neutron fluence (dashed lines). The plot to the left yields the following penalties: InGaAs: -5.1 dB GaAs: -9.6 dB |
| --- | --- |
|  | Worst case is for 6×1015 cm-2 neutron fluence (solid lines). At this fluence we have around 100 µA leakage current for 1.0 V reverse bias (conservative GBTIA value). This yields a 0.3 dB penalty to be added due to PIN leakage current. |

11.2.2. Fiber degradation with cumulative radiation at a glance

Detail results are discussed in reference [7]

Fit function to calculate the total RIA